BLE Pedometer

Communication Protocol Specification

Version: R38

Date: 2015.11.16

Protocol Description

This Agreement is only effective for J-Style BLE Pedometer communication protocol interpretation.

# 1.BLE Profile Description

Service UUID: 0xFFF0

Feature UUID：

TX UUID：0xFFF6   data is sent from smart phone to pedometer, the length is 16 bytes.

RX UUID：0xFFF7   data is sent from pedometer to smart phone, the length is 16 bytes.

BLE communication is the interaction only between two feature UUID, the specific protocol format ,

please read Section 2

# 2.BLE Protocol

A).The PC send16-byte packets to the device, then the device sent back the 16-byte packets. Except for some

special commands, all others can use this communication mode.

B).The PC send the 16-byte packets, the packet format as below:

| B1 | B2 --------------- B15 | B16 |
| --- | --- | --- |
| command | Payload bytes, 14 bytes | CRC checksum byte |

Command：value range: 0x00 0x7F; Bit7 for 0 all the time.

The last byte is CRC，CRC calculation is simple to add the first 15 bytes, and then get the lowest 8 bytes,

for example:

If the packets as below:

A1 A2 A3 A4...... A13 A14 A15 CRC，and

CRC = (A1 + A2 + A3 + A4+……+ A13 + A14 + A15) & 0xFF;

C).After the device get command, then return 16-byte response packets, the format as below:

| B1 | B2 --------------- B15 | B16 |
| --- | --- | --- |
| Command | Payload bytes, 14 bytes | CRC checksum byte |

Command：Response to a received command

For example: If get recommand 0x01, and also CRC check OK, then return 0x01.

If CRC error, please set Bit7 to 1, return 0x81.

The last byte is CRC，CRC calculation is simple to add the first 15 bytes, and then get the

lowest 8 bytes,

for example, the packet:

A1 A2 A3 A4...... A13 A14 A15 CRC，and

CRC = (A1 + A2 + A3 + A4+……+ A13 + A14 + A15) & 0xFF;

# 3.BLE Data Format

1. Set time

Command Format： 	0x01 AA BB CC DD EE FF 00 00 00 00 00 00 00 00 CRC

Function：		Write  APP current time into the device, totally 16bytes

Description：		AA year；BB month；CC day；DD hour；EE minute；FF second。format is BCD，for example,12 year，AA = 0x12

Command response：

Check right and execute OK, then return:：	0x01 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Check error and execute Fail, then return：	0x81 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

2. Get Time

Command Format：	0x41 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：                              PC get current time from the device, total 16 bytes.

Command response：

Check right and execute OK, then return: 0x41 AA BB CC DD EE FF 00 00 00 00 00 00 00 00 CRC

Check error and execute Fail, then return: 0xC1 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Response description：AA year；BB month；CC day；DD Hour；EE Minute；FF Second. The format is BCD，

For example: 12 year，AA = 0x12

3. Set Personal Information：

Command Format：0x02 AA BB CC DD EE 00 00 00 00 00 00 00 00 00 CRC

Function：Set personal information to the pedometer

Description： AA:gender（0 stands for female，1 stands for male），BB:age，CC:height，DD:weight，

EE:stride length；

Command response：

Check right and execute OK, then return: 0x02 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Check error and execute Fail, then return:0x82 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

4. Get user personal setting information:

Command Format： 0x42 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function： PC read user personal information from device.

Command response：

Check right and execute OK, then return: 0x42 AA BB CC DD EE FF GG HH II JJ KK 00 00 00 CRC

Check error and execute Fail, then return:  0xC2 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Response description：AA:gender（0 stands for female，1 stands for male），BB:age，CC:Height，DD:Weight，EE:stride length，FFKK is the device ID code in 6 bytes, high bytes in front.

5. Get detailed current day’s activity data：

Command format： 0x43 AA 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：read the device detailed current day activity data;

Description：AA stands for data of the specified day, 0 stands for current day’s data,

1 stands for data of 1 day ago, 2 stands for data of 2 day ago,and so on，.....29 stands for data of 29

day ago, the device with 30 days memory.

Command response：

Check right and execute OK, then return:

The first response(with data)：0x43 0xF0 AA BB CC DD EE FF GG HH II JJ KK LL MM CRC（total 96 responses）

The first response(without data)：0x43 0xFF 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC（only 1 response）

Check error and execute Fail: 0xC3 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC（only 1 response）

96 pieces data responses, description as below:

AA BB CC:  stands for Year, Month, Day.

DD stands for time scale index, for example:

00:0000:15 stands for the zeroth time scale, the index value is 0.

23:4524:00 stands for the 95th time scale. The Maximum value is 95.

EE: If EE= 0x00 stands for activity data; If EE =0xFF stands for sleep quality monitor data.

The activity data format as below:

FF GG stands for calorie, 2 bytes, low byte in the front.(The calorie be divided by 100 and reserve two decimal)

HH II stands for steps, 2 bytes, low byte in the front.

JJ KK stands for active time, 2 bytes, low byte in the front, and its unit is second..

6. Delete activity data:

Command format：	0x04 AA 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：	                Delete someday’s activity data

- Description：	                AA stands for few days ago’s activity data,0 stands for current day’s activity data.1 stands for one

day ago’s  activity data, the maximum is 29.

Command response：

Check right and execute OK, then return:	0x04 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Check error and execute Fail, then return:	0x84 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

7.Set device ID Code:

Command format：	0x05 ID5 ID4 ID3 ID2 ID1 ID0 00 00 00 00 00 00 00 00 CRC

Function：		PC write ID code to the device

Description：		6-byte ID code arrangement is high byte first

Command response：

Check right and execute OK, then return:	0x05 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Check error and execute Fail, then return:	0x85 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

8. check data storage distribution:

Command format：	46 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：		Check if there is activity data stored within 30 days

Command description：

Check right and execute OK, then return:	0x46 AA BB CC DD 00 00 00 00 00 00 00 00 00 00 CRC

Check right and execute OK, then return:	0xC6 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Command response:

AA BB CC DD for a 4 bytes of data，AA is the highest byte, DD is the lowest byte.

Bit0 corresponds to current day, if current day with data storage then Bit0 for 1, if current day without data storage then Bit0 for 0.

Bit1 corresponds to the day before, Bit29 corresponds to the previous 29 days’data(29 days’ago data).

Bit30-Bit31 for 0 all the time.

9.Read someday’s total activity data

Command format：	0x07 AA 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：		Read somedays’s total activity data

Description： 		AA = 0 stands for current day, AA = 10 stands for 10 days ago

Commnand response:

Check right and execute OK, then return:	0x07 AA BB CC DD EE FF GG HH II JJ KK LL MM NN CRC

Check error and execute Fail, then return:	0x87 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Respond description:( total 2 responses, and each one the first five bytes of the same meaning)

AA stands for the command index values,this command will response two items，the first index values is 0，

The second index values is 1.

BB stands for few days ago’s data.

CC DD EE stands for year month day. (If year, month, day all is 0, it means no data this day)

The first response:

FF GG HH stands for the total steps.

II JJ KK stands for 3-byte running steps/ aerobic steps, high byte in the front.

LL MM NN stands for 3-byte calorie value, high byte in the front.

The second response:

FF GG HH stands for 3-byte walking distance, high byte in the front.

II JJ stands for 2-byte activity time, the unit is minute,

KK LL 00 00

10.Read someday’s activity goal achieved rate:

Command format：                 0x08 AA 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：		  Read someday’s total activity data

Description： 		  AA = 0 stands for current day, AA = 10 stands for 10 days ago

Command response：

Check right and execute OK, then return:	0x08 AA BB CC DD EE FF GG HH II JJ KK LL MM NN CRC

Check right and execute OK, then return:	0x88 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

AA stands for days ago’s data, 0 stands for current day, 1 stands for the day before.

BB CC DD stands for year,month, day. (If year, month, day all is 0, it means no data this day)

EE 00, can ignore

FF GG stands for 2-byte activity speed, high byte in the front.

HH II JJ stands for 3-byte EX value, high byte in the front.

KK LL stands for personal goal finish percentage, 2 bytes, high byte in the front. Value range 0  999

MM NN 00 00, can ignore

11. Start up step real-time meter mode

Command format:             0x09 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：	         Start up step real-time meter mode

Command response:

Check right and execute OK, then return:	0x09 AA BB CC DD EE FF GG HH II JJ KK LL MM NN CRC

Check error and execute Fail then return:	0x89 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Response description:

After start up succeed, and if the pedometer data with some change, it will sent the command response again

Automatically

AA BB CC stands for 3-byte total steps, AA stands for the highest byte, CC stands for the lowest byte.

DD EE FF stands for 3-byte running steps/aerobic steps, DD stands for the highest byte, FF stands for the lowest byte.

GG HH II stands for 3-byte calorie value, GG stands for the highest byte, II stands for the lowest byte.

JJ KK LL stands for 3-byte walking distance, JJ stands for the highest byte, LL stands for the lowest byte.

MM NN stands for 2-byte activity activity time, the unit is minute.

12. Stop step real-time meter mode:

Command format： 	0x0A 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：		Stop step real-time meter mode

Stop step real-time meter mode, and then the pedometer will stop automatically to uploading real-time activity data.

Command response：

Check right and execute OK, then return:	0x0A 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Check error and execute Fail, then return:	0x8A 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

13. Set target steps ：

Command format： 	0x0B AA BB CC 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：	                Step input target steps to the pedometer

Description：	                AA BB CC is the target steps value，3 bytes，high byte in front.

Command response：

Check right and execute OK, then return:	0x0B 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Check error and execute Fail, then return:	0x8B 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

14. Get the target steps ：

Command format： 	0x4B 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：		Smartphone  read target steps on the device

Command response：

Check right and execute OK, then return:                0x4B 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Check error and execute Fail, then return:	0xCB AA BB CC 00 00 00 00 00 00 00 00 00 00 00 CRC

Response description ：    AA BB CC is the target steps value，3 bytes，high byte in front.

15. Read device battery：

Command format： 		0x13 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：			Read device battery

Description：

Command response：

Check right and execute OK, then return:：	0x13 AA 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Check error and execute Fail, then return：	0x93 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Response description：AA  battery level，from 0x00 to 0x08, 0x08 is for the highest battery level, 0x00 is for the lowest battery level。

16. Device Bonding：

Command format： 0x20 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：can bond the device when bluetooth connected

Description：

Command response：

Check right and execute OK, then return:：0x20 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Check error and execute Fail, then return：0xA0 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

17. Answer to confirm the Bonding：

Command format： 0x21 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：When device gets the bonding command, will confirm this command if user tap device within 10 seconds

Description：

Command response：

Check right and execute OK, then return:：0x21 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Check error and execute Fail, then return：0xA1 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

18. Read MAC address：

Command format： 0x22 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：Read the device MAC add

Description：

Command response：

Check right and execute OK, then return:：0x22 MAC0 MAC1 MAC2 MAC3 MAC4 MAC5 00 00 00 00 00 00 00 00 CRC

Check error and execute Fail, then return：0xA2 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

19. Read software version number :

Command format:    0x27 00 00 00 DD 00 00 00 00 00 00 00 00 00 00 CRC

Function:                  Read software version number

Command response：

Check right and execute OK, then return:                     0x27 AA BB CC DD 00 00 00 00 00 00 00 00 00 00 CRC

Check wrong then execute Fail then return:                  0xA7 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Description:

AA BB CC DD are software version number(16 bytes BCD form)

20. MCU reset command:

Command format:              0x2E 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function:                            APP reset and control MCU

Description:

Command response：

Check right and execute OK, then return:                     0x2E 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Check wrong then execute Fail then return:                 0xAE 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

21. Convey Motor vibration signals:

Command format:            0x36 AA 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function:                      Set display mode

Description:       AA: vibration time, unit is 0.25ms. The max value is 10, that is 2.5s.

Command response：

Check right and execute OK, then return:  0x36 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Check wrong then execute Fail then return:    0xB6 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

22. Set time format

Command format： 0x37 AA 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：set time format

Description：AA： =0 stands for 12hour format, =1 stands for 24hour format

Command response：

Check right and execute OK, then return：0x37 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Check wrong then execute Fail then return：0xB7 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

23. Read time format

Command format： 0x38 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：Read time format

Command format：

Check right and execute OK, then return：：0x38 AA 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Check wrong then execute Fail then return：0xB8 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Response Description: AA： =0  stand for 12hour format；=1 stands for 24 hour format

24. Get Current Activity info

Command format： 		0x48 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：			Read/Upload current activity info to app

Command response：

Check right and execute OK, then return：	0x48 AA BB CC DD EE FF GG HH II JJ KK LL MM NN CRC

Check wrong then execute Fail then return：	0xC8 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Response Description：

AA BB CC stands for 3-byte total steps, AA stands for the highest byte，CC stands for the lowest byte。

DD EE FF stand for 3-byte running steps, DD stands for the highest byte, Ffstandards for the lowest byte

GG HH II stands for 3-byte calorie value, GG stands for the highest byte, II stand for the loweset byte（The calorie be divided by 100 and reserve two decimal）

JJ KK LL stands for 3-byte distance, JJ stands for the highest byte, LL stands for the lowest byte (Distance unit is 0.01KM)

MM NN stands for 2-byte activity time, unit is minute

25. Set user name
Command format： 0x66 ID BB CC DD EE FF GG HH II JJ KK LL MM NN CRC  
Function：Set user name
ID：No，1st   pc is 1, and 2nd pc  is 2.   If user name length is not more than 12 characters, only need one command.
BB：User name length。
CC DD EE FF GG HH II JJ KK LL MM NN：User name content：ASCII code.
Command response：
Check right and execute OK, then return：0x66 AA 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC
AA:ID
Check wrong then execute Fail then return：0xE6 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Description：AA BB CC DD EE FF GG HH II JJ KK LL  is ASCII code, must be English capital letters，dash, 


26. Get user name

Command format： 0x67 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC  
Function：Get user name
Command response：      Return user name  character string
Check right and execute OK, then return：0x67 ID BB CC DD EE FF GG HH II JJ KK LL MM NN CRC
ID：No, 1st pc is 1, 2nd pc is 2. If user name is not more than 12, only need one command
BB：User name length。
CC DD EE FF GG HH II JJ KK LL MM NN：User name content：ASCII code
Command response：
Check right and execute OK, then return：0x66 AA 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC
AA:ID
Check wrong then execute Fail then return：0xE7 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

- Command format：： 0x66 AA BB CC DD EE FF GG HH II JJ KK LL 00 00 CRC 
Function： set User name 
Command response： 
Check right and execute OK, then return：：0x66 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 
Check wrong then execute Fail , then return：：0xE8 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 

Description：AA BB CC DD EE FF GG HH II JJ KK LL  is ASCII  code, must be English uppercase letters, and slash.

26. Get user name

Comman format： 0x67 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC  
Function：Get user name
Command response：  Return to user name  chracter string
Check right and execute OK, then return：0x67 AA BB CC 00 00 00 00 00 00 00 00 00 00 00 CRC
Check wrong then execute Fail , then return：0xE7 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

27. Set Inspired message

Command format： 0x67 AA BB CC DD EE FF GG HH II JJ KK LL 00 00 CRC 
Fuction： Set inspired message 

Description：AA: inspire and Index，value range 0-3 
BB CC DD EE FF GG HH II JJ KK LL MM  is inspire message’s  ASCII code，must be English uppercase letters, and  slash

Command response： 
Check right and execute OK, then return：：0x67 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 
Check wrong then execute Fail , then return：：0xE7 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

28.  Get inspiring message
Command format： 0x69 AA 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC  
Function：Read inspiring message

Command response：return to some inspiring message chracter string
Description：AA: inspiring index，value range 0-3 , which means to get some inspiring message
Check right and execute OK, then return：0x69 AA BB CC DD EE FF GG HH II JJ KK LL MM 00 00 CRC
Check wrong then execute Fail , then return：0xE9 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

29. Set Calories Mission goal
Command format： 0x70 AA BB CC 00 00 00 00 00 00 00 00 00 00 00 00 CRC  
Function：Set calories mission goal

Description：AA:  1: Set goal     2：change goal     3：turn off goal

BB CC calorie goal data, high byte in front.

Command response：
Check right and execute OK, then return：0x70 aa 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC
：0xE0 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

30. Get calorie mission goal
Command format： 0x71  00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC  
Function：Get calories mission goal

Command response：
Check right and execute OK, then return： 0x71 AA BB CC DD EE FF GG HH II 00 00 00 00 00 00 00 CRC

Description：AA:  1: Enable calorie mission goal    0：turn off calorie mission goal

BB CC DD EE  calorie goal data, high byte in front

FF GG HH II    total calories

JJ KK LL MM   current cumulative calories consumed.

Check wrong then execute Fail , then return：0xE2 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

31. Set a static image
Command format： 0x72 AA BB CC DD EE FF GG HH II  JJ KK LL MM  00 CRC  
Function：Set a static image

Description：AA  is number

BB CC DD EE FF GG HH II  JJ KK LL MM is image

Detailed description for data

- Relations between byte and dot matrix.

- Our 128 * 32 OLED screen is displayed in black and white, no gray levels. That is only two states at any one point, either white (not shown) or black (black display). So long as one bit of data can be shown to a point, that is, 0 or 1. Then one byte of data can represent eight points OLED screen. Therefore, the general OLED screen width and height of the total number of points is a multiple of eight. For example 128X32.

Each byte point sequency on the OLED screen

That will be a byte of data to the OLED module, it can display eight points respectively, but how to arrange eight points?

Starting from the first row to take eight points as a byte, and then start from the second row to take eight points as the second byte ... and so on. If the last less than eight points , make up the full eight. Modulo order from low to high, the first point as the lowest bit.  For example , ------- Taken as 00000001

Command response：
Check right and execute Ok, then return：0x72 aa 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC
Check wrong and execute fail and return：0xE2 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

32. Send animated celebration display to device

Command format：      0x73 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function：		Send animated celebration display to device

Command response：

Check right and execute OK and return：	0x73 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Check wrong and execute Fail and return：	0xE3 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC